Foam tail of the Topsky DLG

installation Guide

- cut the frame from wood board

- use CA to built the tail installation frame. R1-R4 for rudder ,and E1-E4 for elevator

- cut a slot about 27x10 mm on the rudder

- sand a little slot in the center at the end of the boom ,about 5mm width.

- pre-connect the boom and rudder, maybe you need to trim the rudder slot width

- tape the paper on the table, and draw a long line

- place the tail frame on the center of the paper

- please notice the slot on the side of  frame must on the paper line

- place the rudder and boom on the frame

- check the following :

  - hinge slot parallel to the paper line

  - downside of the airfoil  fit the frame very good

  - boom is parallel to the long line on the table

- use the 6 minuts glue

- use a little glue to fix the boom and rudder

- after several minutes , fix another side of the rudder with a little 6 minutes glue

- sand the downside of the elevator platform

- sand the upside of the elevator platform

- press some hole inboard of the carbon with knife

- pre-install the boom and platform and elevator, and check them fit or not

- draw a vertical line on the table

- place the elevator frame on center of the paper

- please notice that the slot on the side of the frame must be on the paper line

- put the elevator on the frame

- pre-install the boom , platform , elevator. Check the fit or not,  and the boom must  parallel to the table.

- use 6 minutes glue fix them

- check the following:

  - upside of the elevator airfoil fit the frame very good

  - rudder is vertical to elevator

  - hinge slut parallel to the paper line

  - boom parallel to the long line on the table and table face.

- after several minutes , cut 3 fiberglass pieces

- use the epoxy

- fix the elevtor

- and fix the rudder

- when the epoxy solidify, cut the unwanted fiberglass