Manual of RC Timer ESC  Brushless Motor Speed Controller

Thanks for purchasing RC Timer Electronic Speed Controller (ESC). High power system for RC model can be very dangerous, so we strongly suggest you read this manual carefully. In that we

have no control over the correct use, installation, application, or maintenance of our products, no liability shall be assumed nor accepted for any damages, losses or costs resulting from the use

of the product. Any claims arising from the operating, failure or malfunctioning etc. will be denied. We assume no liability for personal injury, property damage or consequential damages

resulting from our product or our workmanship. As far as is legally permitted, the obligation to compensation is limited to the invoice amount of the affected product.

Specification

Input voltage: DC 6-16.8V(2-4S Lixx)

Running current:30A(Output: Continuous 30A, Burst 40A up to 10 Secs.)

Size: 26mm (L) * 23mm (W) * 11mm (H).

Weight: 32g.

Features

Extreme low output resistance, super current endurance.

Multiple protection features: Low-voltage cut-off protection / over-heat protection / throttle signal loss protection.

3 start modes: Normal / Soft / Super-Soft, compatible with fixed-wing aircraft and helicopter.

Throttle range can be configured to be compatible with all transmitters currently available on market.

Smooth, linear and precise throttle response.

Separate voltage regulator IC for microprocessor (except Pentium-6A and Pentium-10A), providing good anti-jamming capability.

Supported motor speed (Maximum): 210000 RPM (2 poles), 70000 RPM (6 poles), 35000 RPM (12 poles).

Wire Diagram

Very important: if you use banana-shape connectors on main power wires (Input wires), please connect the black wire (negative polarity) BEFORE red wire (positive polarity). So the right

Page 2

sequence is: BDMP Adapter → BLACK wire of main power → RED wire of main power

Programmable Items:

1. Brake Setting：Enabled / Disabled, default is Disabled

2. Battery Type：Li-xx(Li-ion or Li-poly) / Ni-xx(NiMH or NiCd)，default is Li-xx.

3. Low Voltage Protection Mode(Cut-Off Mode)： Soft Cut-Off (Gradually reduce the output power) or Cut-Off (Immediately stop the output power). Default is Soft Cut-Off.

4. Low Voltage Protection Threshold(Cut-Off Threshold)：Low / Medium / High, default is Medium.

1) For lithium batteries, the number of battery cells is calculated automatically. Low / medium / high cutoff voltage for each cell is: 2.6V/2.85V/3.1V. For example: For a 3 cells lithium pack,

when “Medium” cutoff threshold is set, the cut-off voltage will be: 2.85*3=8.55V.

2) For nickel batteries, low / medium / high cutoff voltages are 0%/45%/60% of the startup voltage (i.e. the initial voltage of battery pack), and 0% means the low voltage cut-off function is

disabled. For example: For a 10 cells NiMH battery, fully charged voltage is 1.44*6=8.64V, when “Medium” cut-off threshold is set, the cut-off voltage will be:8.64*50%=4.3V。

5. Startup Mode：Normal /Soft /Super-Soft, default is Normal.

Normal is preferred for fixed-wing aircraft. Soft or Super-soft are preferred for helicopters. The initial acceleration of the Soft and Super-Soft modes are slower in comparison, usually taking 1

second for Soft startup or 2 seconds for Super-Soft startup from initial throttle advance to full throttle. If the throttle is closed (throttle stick moved to bottom) and opened again (throttle stick

moved to top) within 3 seconds of the initial startup, the restart-up will be temporarily changed to normal mode to get rid of the chances of a crash caused by slow throttle response. This special

design is very suitable for aerobatic flight when quick throttle response is needed.

6. Timing：Low / Medium / High, default is Low.

Usually, low timing value can be used for most motors. We recommend the Low timing value for 2 poles motor and Medium timing value for motors with more than 6 poles to get a high efficiency.

For higher speed, High timing value can be chosen.

Special Note

Some high KV out-runner motors have very special construction, the space between each magnet is very large, and many ESCs can’t drive these motors. After much testing, our ESCs have

proven to work very well with these types of motors. Therefore, we have provided some suggestions as follows:

Programmable

Motor

Timing

Startup mode

Generic in-runner motor

Low

Generic out-runner motor

Low or Medium

Align 420LF（Made in TAIWAN，out-runner）

High（MUST）

Usually,aircraft use “Normal” startup mode and helicopter

use”super-soft” startup mode

450TH（Made in TAIWAN）

Low

Soft(MUST

Begin To Use Your New ESC

Please start the ESC in the following sequences:

1. Move the throttle stick to the bottom position and then switch on the transmitter.

2. Connect the battery pack to the ESC, the ESC begins the self-test process, a special tone “ 123” is emitted, which means the voltage of the battery pack is in normal range, and then N “beep”

tones will be emitted, means the number of lithium battery cells.Finally a long “beep------” tone will be emitted, which means self-test is OK, the aircraft/

If a special tone “ 56712” is emitted after 2 beep tones (“beep-beep-”), means the ESC has entered the program mode, it is because the throttle channel of your transmitter is reversed, please

set it correctly;

Page 3

If the very rapid “beep- beep-, beep-beep-” tones is emitted, means the input voltage is too low or too high, please check your battery’s voltage.

Normal startup procedure:

Move throttle

s t i c k t o

bottom and

then swi tch

on transmitter.

Connect battery

p a c k t o E S C ,

special tone like

“ 1 2 3 ” m e a n s

power supply is OK

Several “beep-” tones

should be emitted,

presenting the number

of lithium battery cells

When self-test is

finished, a long

“beep-----”tone

should be emitted

Move throttle stick

upwards to go flying

“VERY IMPORTANT !” Because different transmitter has different throttle range, we strongly suggest you using the “Throttle Range Setting Function” to calibrate throttle range. Please

read the instruction on page 4------“Throttle Range Setting”.

Throttle range setting: (Throttle range should be reset whenever a new transmitter is being used)

Swi tch on

transmitter,

move throttle

stick to top

Connect battery

pack to ESC,

and wait for

about 2 seconds

“Beep-Beep-” tone

should be emitted means throttle

range highest point has been

correctly confirmed

Move throttle stick to the

bottom, several “beep-” tones

should be emitted, presenting

the number of battery cells

A long “Beep-” tone should

be emitted, means throttle

range lowest point has

been correctly confirmed

Alert Tone

1. Input voltage is abnormal: The ESC begins to check the voltage when the battery pack is connected, if the voltage is not in the acceptable range, such an alert tone will be emitted:

“beep-beep-, beep-beep-,beep-beep-” (Every “beep-beep-” has a time interval of about 1 second. )

2. Throttle signal is abnormal: When the ESC can’t detect the normal throttle signal, such an alert tone will be emitted: “beep-, beep-, beep-”. (Every “beep-” has a time interval of about 2

seconds)

3. Throttle stick is not in the bottom position: When the throttle stick is not in bottom (lowest) position, a very rapid alert tone will be emitted: “beep-, beep-, beep-”. (Every “beep-” has a time

interval of about 0.25 second.)

Protection Function

1. Abnormal start up protection: If the motor fails to start within 2 seconds of throttle application, the ESC will cut-off the output power. In this case, the throttle stick MUST be moved to the

bottom again to restart the motor. (Such a situation happens in the following cases: The connection between ESC and motor is not reliable, the propeller or the motor is blocked, the gearbox is

damaged, etc.)

2. Over-heat protection: When the temperature of the ESC is over 110 Celsius degrees, the ESC will reduce the output power.

3. Throttle signal loss protection: The ESC will reduce the output power if throttle signal is lost for 1 second, further loss for 2 seconds will cause its output to be cut-off completely.

Page 4

Program the ESC with your transmitter (4 Steps):

1. Enter program mode

2. Select programmable items

3. Set item’s value (Programmable value)

4. Exit program mode

1. Enter program mode

1) Switch on transmitter, move throttle stick to

top , connect the battery pack to ESC

2) Wait for 2 seconds, the motor should emit

special tone like “beep-beep-”

3) Wait for another 5 seconds, special tone like

“56712 ” should be emitted, which means

program mode is entered

4. Exit program mode

There are 2 ways to exit

program mode:

1. In step 3, after special tone

“1515 ”, please move throttle

stick to the bottom position

within 2 seconds.

2. In step 2, after tone

“beep-----beep-----”(ie. The item

#8), move throttle stick to

bottom within 3 seconds.

3. Set item value (Programmable value):

You will hear several tones in loop. Set the value matching to a tone by moving throttle stick to top when you hear the tone,

then a special tone “1515 ” emits, means the value is set and saved. (Keeping the throttle stick at top, you will go back to

step 2 and you can select other items; Moving the stick to bottom within 2 seconds will exit program mode directly)

Tones

Items

“beep-”

1 short tone

“beep-beep-”

2 short tones

“beep-beep-beep”

3 short tones

Brake

off

on

Battery type

Li-ion / Li-poly

NiMH / NiCd

Cutoff mode

Soft-Cut

Cut-off

Cutoff threshold

Low

Medium

High

Start mode

Normal

Soft

Super soft

Timing

Low

Medium

High

2. Select programmable items:

After entering program mode, you will hear 8 tones in a loop with

the following sequence. If you move the throttle stick to bottom

within 3 seconds after one kind of tones, this item will be selected.

1. “beep” brake (1 short tone)

2. “beep-beep-” battery type (2 short tone)

3. “beep-beep-beep-” cutoff mode (3 short tone)

4. “beep-beep-beep-beep-” cutoff threshold (4 short tone)

5. “beep-----” startup mode (1 long tone)

6. “beep-----beep-” timing (1 long 1 short)

7. “beep-----beep-beep-” set all to default (1 long 2 short)

8. “beep-----beep-----” exit (2 long tone)

Note: 1 long “beep-----” = 5 short “beep-”

Page 5

Program Example

Setting “Start Mode” to “Super-Soft”, i.e. value #3 in the programmable item #5

1. Enter Program Mode：Switch on transmitter, move throttle stick to top position, connect battery pack to ESC, wait for 2 seconds, “beep-beep” tone should be emitted. Then wait for another 5

seconds, special tone like “56712 ” should be emitted, which means program mode is entered.

2. Select Programmable Items：Now you’ll hear 8 tones in a loop. When a long “beep------” tone is emitted, move throttle stick to bottom to enter the “Start Mode”

3. Set Item Value (Programmable Value)：“Beep-”, wait for 3 seconds; “Beep-beep-”, wait for another 3 seconds; then you’ll hear “beep-beep-beep”, move throttle stick to top position, then a

special tone “1515 ” is emitted, now you have set the “Start Mode” item to the value of “Super-Soft”

4. Exit Program Mode：After the special tone “ ”, move throttle stick to bottom within 2 seconds.

Trouble Shooting

Trouble

Possible Reason

Action

After power on, motor does not work, no sound is emitted

The connection between battery

pack and ESC is not correct

Check the power connection. Replace the connector.

After power on, motor does not work, such an alert tone is

emitted:“beep-beep-,

beep-beep-,beep-beep-”

(Every

“beep-beep-” has a time interval of about 1 second)

Input voltage is abnormal, too high

or too low.The balance charge connector is

not located properly in BDMP adapter.

Check the voltage of battery pack Check the connection of the balance

charge connector and the BDMP adapter.

After power on, motor does not work, such an alert tone is

emitted:“beep-, beep-, beep- ”(Every “beep-” hasa time

interval of about 2 seconds)

Throttle signal is irregular

Check the receiver and transmitter

Check the cable of throttle channel

After power on, motor does not work, such an alert tone is

emitted:“beep-, beep-, beep-” (Every “beep-” has

a time interval of about 0.25 second)

The throttle stick is not in the

bottom (lowest) position

Move the throttle stick to bottom position

After power on, motor does not work, a special tone “56712 ” is

emitted after 2 beep tone (beep-beep-)

Direction of the throttle channel is reversed, so

the ESC has entered the program mode

Set the direction of throttle channel correctly

The motor runs in the opposite direction

The connection between ESC and the motor

need to be changed.

Swap any two wire connections between ESC and motor

Throttle signal is lost

Check the receiver and transmitter Check the cable of throttle channel

ESC has entered Low Voltage Protection mode

Land RC model as soon as possible, and then replace the battery pack

The motor stop running while in working state

Some connections are not reliable

Check all the connections: battery pack connection, throttle signal cable,

motor connections, etc.

Random stop or restart or irregular working state

There is strong electro-magnetic interference in

flying field.

Reset the ESC to resume normal operation. If the

function could not resume, you might need to move to another area to fly.

Page 6